PRACTICAL 5 : Temperature on LCD

Aim : To interface a 16x2 LCD with Arduino and display temperature readings.

Overview :

In this project, a temperature sensor is interfaced with an Arduino and a 16x2 LCD to display real-time temperature readings. This practical helps in learning about LCD interfacing, data communication between components, and the importance of real-time monitoring in IoT systems.

Materials Required :

- Arduino Uno R3

- 1 x 220 Ω Resistor

- LCD 16x2

- Temperature Sensor (TMP36)

- Jumper Wires

- Arduino IDE (Installed on your Computer)

Circuit Connection and Steps :

- Connect the TMP36 Temperature Sensor :

  - VCC → 5V, GND → GND, VOUT → A5 (Arduino).

- Connect the 16x2 LCD Display :

  - RS → D7, E → D8, D4-D7 → D9-D12 (Arduino).

  - VSS, RW, K → GND, VDD, A → 5V.

  - V0 → Potentiometer (Middle Pin) for contrast control.

- Connect the Potentiometer:

  - One side → 5V, Other side → GND, Middle → V0 (LCD).

- Power Connections:

  - 5V → Positive Rail, GND → Negative Rail on Breadboard.

Circuit Diagram :

Schematic Diagram :

Code :

#include<LiquidCrystal.h>

const int rs = 12, en = 11, d4 = 5, d5 = 4, d6 = 3, d7 = 2;

LiquidCrystal lcd(rs, en, d4, d5, d6, d7);

float celsius;

int temp = A1;

void setup(){

pinMode(temp,INPUT);

}

void loop(){

celsius = analogRead(temp)*0.004882814;

celsius = (celsius - 0.5) * 100.0;

lcd.setCursor(0,1);

lcd.print("--> Temp : ");

lcd.print(celsius);

lcd.print(" C");

delay(1000);

lcd.clear();

}

Results :

Conclusion :

This project successfully demonstrates how to interface a 16x2 LCD with an Arduino to display real-time temperature readings. It reinforces the concepts of sensor integration and data visualization, providing a crucial step toward developing more complex IoT applications involving real-time monitoring systems.