PRACTICAL 9 : Light, Humidity & Temperature Monitoring

Aim : To measure and display light intensity, humidity and temperature using respective sensors.

Overview :

This project integrates multiple sensors to measure light intensity, humidity and temperature. The collected data is displayed on an output device. It introduces multi-sensor interfacing and demonstrates how environmental monitoring can be implemented in IoT applications.

Materials Required :

- Arduino Uno R3

- 250 kΩ Potentiometer

- Temperature Sensor (TMP36)

- Ambient Light Sensor (Phototransistor)

- 3 x 1 kΩ Resistor

- Wifi Module (ESP8266)

- Breadboard Small

- Jumper Wires

- Arduino IDE (Installed on your Computer)

Circuit Connection and Steps :

- Powering Components : Connect the VCC of all sensors and the ESP8266 to 5V, except the ESP8266 (VCC & CH_PD → 3.3V). Ensure all GND pins are connected to a common ground.

- Sensor Connections :

  - The LM35 temperature sensor outputs data from its OUT pin to A0 of the Arduino.

  - The humidity sensor (potentiometer) has its middle pin connected to A1 for analog readings.

  - The LDR (light sensor) connects one leg to A2, the other to 5V, with a 10kΩ pull-down resistor to GND for stability.

- ESP8266 Communication :

  - Connect the ESP8266 TX to Arduino RX through a level shifter for proper voltage handling.

  - Connect the ESP8266 RX to Arduino TX directly.

  - Pull CH_PD high by connecting it to 3.3V to enable the module.

Circuit Diagram :

Schematic Diagram :

Code :

// WiFi Credentials

String ssid = "Simulator Wifi";

String password = "";

// ThingSpeak API Configuration

String host = "api.thingspeak.com";

const int httpPort = 80;

String apiKey = "YE76IRAW2UVM7LI5";

String endpoint = "/update?api_key=" + apiKey;

// Sensor Pin Configuration

const int tempPin = A0;

const int humidityPin = A1;

const int lightPin = A2;

void setup() {

Serial.begin(115200);

Serial.println("AT");

delay(500);

// Connect to WiFi

Serial.println("AT+CWJAP=\"" + ssid + "\",\"" + password + "\"");

delay(500);

// Establish TCP Connection

Serial.println("AT+CIPSTART=\"TCP\",\"" + host + "\"," + String(httpPort));

delay(500);

}

void loop() {

// Read sensor data

float temperature = ((analogRead(tempPin) * 0.0048828125) - 0.5) * 100;

float humidity = map(analogRead(humidityPin), 0, 1023, 0, 100);

float lightIntensity = map(analogRead(lightPin), 0, 471, 0, 100);

// Construct HTTP GET request

String httpRequest = "GET " + endpoint + "&field1=" + String(lightIntensity) +

"&field2=" + String(temperature) + "&field3=" + String(humidity) +

" HTTP/1.1\r\nHOST: " + host + "\r\n";

// Send data to ThingSpeak

Serial.println("AT+CIPSEND=" + String(httpRequest.length()));

delay(500);

Serial.println(httpRequest);

delay(1000);

}

Results :

Conclusion :

The multi-sensor project successfully measures light intensity, humidity and temperature, showcasing the integration of multiple sensors with Arduino. It emphasizes real-time environmental data collection, which is crucial for applications in smart agriculture, weather monitoring and automated control systems.